# Design Specification for Floating Point Unit of ALU

- Some Assumptions derived from Function Specification::

- All designs are complied to MIPS32

- Only Support IEEE 574 format (single precision)

- FPU is a core part of CPU, not stand-alone

- The FPU has no own storage registers, all values and results should be stored in the CPU.

- The FPU core assumes that the CPU has to issue the instructions and know when the result should come out

- The CPU should decide whether can issue FP instructions or not (detect hazards etc.).

- Instruction Set Design

Arithmetic Instructions  Supported:

| Mnemonic | Instruction |
| --- | --- |
| ADD.fmt | Floating Point Add |
| SUB.fmt | Floating Point Subtract |
| MUL.fmt | Floating Point Multiply |
| DIV.fmt | Floating Point Divide |
| ABS.fmt | Floating Point Absolute Value |

FPU Instruction Formats:

Note: Only support register operands

6                       5                        5                           5                      5                   6

| COP1 t | fmt | ft | fs | fd | function |
| --- | --- | --- | --- | --- | --- |

Function value of Arithmetic Instruction:

| Mnemonic | Instruction | function value: |
| --- | --- | --- |
| ADD | Add | 000000 |
| SUB | subtract | 000001 |
| MUL | multiply | 000010 |
| DIV | divide | 000011 |
| ABS | Absolute value | 000101 |

  - As all the function values begin with 3 zeros, only last 3-bits make sense.

IEEE 574 Format (only “single” format is supported):

1       8                                              23

| S | Exponent | Fraction |
| --- | --- | --- |

- External Interface Desgin

Requiremetns:

- Two operands are input from data-path

- Operation type is inputted from Control path.

- Result is output to data-path.

- The unit should give out exceptions .

- The unit should give “ready” signal when finished in the Control Path

| Deriction | Size | Meaning |
| --- | --- | --- |
| INPUT | [31:0] | Operand A |
| INPUT | [31:0] | Operand B |
| INPUT | [2:0] | Operation Type |
| OUTPUT | [31:0] | Result |
| OUTPUT | 1 | Exception (Overflow, Underflow, Divide by Zero) |
| OUTPUT | 1 | READY |

- The Top-level Diagram of the Floating Point Unit

The top-level of the FPU design is shown in the figure below. Preceding all the functional units is a pre-decode block. The pre-decode block decode each input. Following the block is a calculation unit, in which all add, sub, absolute, multiply and divide operations are executed.  Actually, the calculation unit is made up of several sub-units such as ADD/SUB, MULT, DIV etc.Following this block are two sets of MUXes. One set selects a data-output from the calculation unit, the other MUX is to select “exception” flag outputs from the calculation unit. The MUXes are sharing the same select lines, so that the data output and flag outputs are always selected from the same sub-units of calculation unit.

XOR

SUB

SUB

A[23:30]

B[23:30]

XOR

A[31]

B[31]

A[0:22]

B[0:22]

Add 127

MUX

Add

SR 2 bit

SR 1 bit

24-bit divider

SL n bit

MSB

n

Add 127

XOR

SUB